EOS-S520 Series

High-quality cooled thermal imager

- With 14-bit digital thermal image output function

- With image tracking function

- With a 20 km laser range finder

- Pan: 360° continuous rotate, Tilt: –10° to +185°

- With a variety of continuous zoom thermal lens

- HD daylight camera with continuous zoom lens

- Can work with radar

- The positioning accuracy is up to 0.003°

- Achieves IP66 protection, suitable for com- plex weather conditions

EOS-S520 series thermal imaging system is a high-performance monitoring platform that integrates thermal imager with daylight camera and laser range finder, and has high protection levels. It can be fixed installing on ground base, vehicle-mounted or shipborne. It can be a stand-alone system or can be integrated into existing net-

works.

Configuration and Features

Thermal Camera

Thermal camera is equipped with cooled 640x512 MCT detector, it gives you clear crisp image with embedded ad- vance image processing technology. Combined with a 110-1100mm or 35-700mm continuous zoom lens, it is suitable for use in dark, complex weather conditions and provides clear thermal image for distant targets.

Daylight Camera

Daylight camera is equipped with 1/1.8” CMOS detector which provides more crisp detail and colorful image than that of 1/3”, it will also have more powerful ability to see target through fog and haze. A 60X optical zoom lens, up to focus 1200mm, is for extreme long range detection.

Pan-Tilt

S520 adopts high precision direct drive motor, which makes the equipment rotate more smoothly, position more ac- curate, response more timely. The positioning accuracy is up to 0.003°.

Image tracking function

Video tracking is based on a powerful video processor with powerful video processing and analysis functions and AI learning capabilities. It can process the video signal, overlay the system status character, and output to the monitor and video recorder through the form of network video stream. Through AI deep learning algorithm, it can realize the function of automatic identification and tracking of various targets. In tracking mode, the platform can automatically turn as the tracked target moves, and the user can select the target to track in the thermal or visible image.

Laser Range Finder

An eye safe Laser Range Finder is equipped in the system to detect the distance of the target up to 20Km.

Technical Specifications

| Model | EOS-520C | EOS-520CL |
| --- | --- | --- |
| Thermal Camera |  |  |
| Detector Type | MCT cooled 640x512 FPA |  |
| Pixel | 15μm |  |
| Spectral Range | 3 - 5μm |  |
| Rate | 50Hz |  |
| NETD | ≤25mk |  |
| Focus Length | 35 - 700mm continuous zoom, 10X | 110 - 1100mm continuous zoom, 10X |
| FOV | 15.6°x 12.5° - 0.79°x 0.63° | 5°x 4° - 0.5°x 0.4° |
| F number | F4 | F5.5 |
| Daylight Camera |  |  |
| Image Sensor | 1/1.8” CMOS |  |
| Resolution | 2048x1536, 3MP | 2688x1520, 4MP |
| Focus Length | 15-775mm continuous zoom, 52X | 20-1200mm continuous zoom, 60X |
| FOV | 26.5°×20° - 0.52°×0.39° | 22.1°×12.6° - 0.37°×0.21° |
| Focus | Auto/Manual/Semi-Auto |  |
| Video Output | Network & LVDS |  |
| Defog | Electronic-Defog / Optical-Defog |  |
| Pan-Tilt |  |  |
| Az Range | n×360° |  |
| EI Range | -10° - +185° |  |
| Maximum angular velocity | 120°/s |  |
| Maximum angular acceleration | 150°/s2 |  |
| Positioning Accuracy | ≤0.005° |  |
| Drive Mode | Direct drive motor, High-speed start and stop |  |
| speed adaptive | Support lens focal length and rotation speed adaptive function |  |
| Preset | Programmable to 255 points |  |
| Scheduled Task | Preset Patrol / Linear Scan / Apple Peel Scan |  |
| Laser Range Finder |  |  |
| Range | 1535±5nm | 1570±5nm |
| Continuous Measuring Rate | 1~10Hz | 1~5Hz |
| Accuracy | ≤±2m（RMS） | ≤±5m（RMS） |
| Range | ≥12km ( for the target 2.3m x 2.3m) | ≥20km ( for the target 2.3m x 2.3m) |

| Tracking |  |
| --- | --- |
| Tracking Video | Support visible/ thermal image video switching tracking |
| Mode | Manual / Auto Tracking |
| Interface |  |
| Video Output | Two channels of network videos and one 14-bit digital thermal image |
| Control Interface | RJ45 / RS422 |
| PTZ Control | Support Pelco-P, Pelco-D |
| Data Return | Can display real-time the azimuth angle and pitch angle of pan-tilt, as well as the focal length value and FOV of daylight camera and thermal camera |
| Power |  |
| Input Voltage | 48VDC±10% |
| Consumption | ≤500W(peak) |
| Environmental Specification |  |
| Operating Temperature | -40℃ ~ +60℃ |
| Storage Temperature | -45℃ ~ +70℃ |
| Humidity | ＜95%RH (no condensation) |
| IP Rating | IP66 |
| Physical Characteristics |  |
| Size | ≤Φ520mm×785mm(H) |
| Weight | ≤95Kg |

Range Performance for Thermal Camera

S520CL（with zoom lens 110mm - 1100mm）

Recognition 9.3Km

Identification 5.4Km

Detection 17.9Km

Recognition 15.3Km

Identification 10.7Km

Detection 21.8Km